PAGE1------------------—----------------------—----------------------—----------------------—----------------------

PID Application

In this training, you will learn:

- What a PID is

- What problems it solves

- How it actually works

- How to implement it in your system

- How to tune the system

- How to model and implement it to MATLAB

PAGE2------------------—----------------------—----------------------—----------------------—----------------------

Who i am

A master 1 student sipnom about drones .

I have a experience in electronics, embedded systems , robotics , and other fields

And i will share it in this formation

PAGE3------------------—----------------------—----------------------—----------------------—----------------------

A PID (Proportional–Integral–Derivative) controller is a control algorithm used to automatically keep a system at a desired value called the setpoint.

In simple terms, a PID continuously:

- Measures the current value of a system (temperature, speed, position, etc.).

- Compare it with the desired value.

- Adjusts the control signal to reduce the difference (called the error).

This process happens repeatedly in a feedback loop, allowing the system to automatically correct itself.

PID controllers are extremely common in real systems, for example:

- Temperature control (ovens, incubators, 3D printers)

- Motor speed control (robots, drones, CNC machines)

- Position control (servo systems)

- Flight stabilization (quadcopters)

- Industrial process control

PAGE4------------------—----------------------—----------------------—----------------------—----------------------

### What Problems Does a PID Solve?

You have an LED and an LDR sensor.
 You want the LED brightness to reach a desired setpoint (a target light level measured by the LDR).

The system must:

- Reach the setpoint as fast as possible

- Maintain the setpoint precisely

- Avoid large deviations, since the system cannot tolerate very high or very low values.

PAGE5------------------—----------------------—----------------------—----------------------—----------------------

You currently think about something like this

if (measured_value < setpoint) {

Turn on ;

}

Else {

turn off;]

As you see this did not work very well . . .

PAGE6------------------—----------------------—----------------------—----------------------—----------------------

Many systems cannot naturally stay at the desired value (setpoint). Disturbances and system dynamics can cause errors.

A PID controller automatically corrects this error and improves the system behavior.

PID mainly solves three problems:

- Slow response: helps the system reach the target faster

- Steady-state error: eliminates the difference between the target and the final value

- Oscillations: stabilizes the system and reduces overshoot

Result: a system that is faster, more accurate, and more stable.

PAGE6------------------—----------------------—----------------------—----------------------—----------------------

### How Does PID Actually Work?

A PID controller works by continuously calculating the error between the desired value (setpoint) and the measured value.

error = setpoint - measured_value
Then it computes the control signal using three terms:

U(t) is the output (the value that we are going to feed to the system)

e(t) is the error

Proportional (P)

- Reacts to the current error.

- Larger error → stronger correction.

Integral (I)

- Reacts to the accumulated past error.

- Eliminates steady-state error.

Derivative (D)

- Reacts to how fast the error is changing.

- Reduces overshoot and improves stability.

PAGE6------------------—----------------------—----------------------—----------------------—----------------------

### How to Implement PID in Your System

It depends on whether the PID controller is implemented in hardware or software.

- Hardware PID: commonly used in industrial controllers (PLCs, dedicated control modules).

- Software PID: implemented inside a microcontroller or computer.
Software PID is widely used in robotics, drones, and embedded systems.

In this training, we will learn how to implement a PID controller in software.

This is how to code the pid :

Yes this its that simple . . .

PAGE7------------------—----------------------—----------------------—----------------------—----------------------

How to tune your system

- Kp​ (Proportional): increases the reaction to the error → makes the system faster, but too much can cause instability.

- Ki​ (Integral): removes steady-state error and helps the system reach the exact setpoint.

- Kd​ (Derivative): reduces overshoot and oscillations, improving stability.

This site allows you to example a PID tunning

PAGE8------------------—----------------------—----------------------—----------------------—----------------------

### How to Model and Implement It in MATLAB

First, we need to identify the system transfer function.

Steps followed:

- Apply an input step

  - I wrote a program that increases the LED command from 0 to 255 (maximum).

- Collect system data

  - The script recorded the LDR sensor values over time.

- Analyze the response

  - I exported the data and used Python to visualize the system response.

- Approximate the system model

  - Using Desmos, I found an approximate mathematical function describing the system behavior.

- Convert to transfer function

  - I applied the Laplace transform to obtain the system transfer function.

- Implement the model in MATLAB

  - The transfer function was implemented in MATLAB.

- Tune the PID controller

  - I used MATLAB’s PID autotuner to compute the optimal (K_p), (K_i), and (K_d) based on the system constraints.

- Apply the parameters to the real system

  - The calculated PID values were implemented in the original code, producing similar results in the real system.

PAGE9------------------—----------------------—----------------------—----------------------—----------------------

Its all about approximation !!

PAGE10------------------—----------------------—----------------------—----------------------—--------------------

Any questions !!

Thanks for participating